GC-MS Non-Target Analysis Pipeline

User Guide

Version 1.0 | January 2026

# Overview

This pipeline automates the processing of Agilent GC-QTOF data from raw instrument files through feature detection and compound annotation. It integrates three major processing stages:

- Conversion: Agilent .D files to mzML format (via ProteoWizard msconvert)

- Processing: Feature detection, spectral deconvolution, and alignment (via MZmine 4)

- Annotation: Level 2 compound identification via spectral library matching

The pipeline can be started from any of these three stages, allowing flexibility for different workflows.

# First-Time Setup

## Prerequisites

Before setting up the pipeline, ensure you have the following installed:

- Python 3.8 or higher (Anaconda distribution recommended)

- ProteoWizard msconvert (included with OpenMS or standalone installation)

- MZmine 4 (portable version, no admin rights required)

## Directory Structure

Create the following directory structure. The default location is:

X:\Data\ChrisB\Non-Target\gcms_pipeline\

Required subdirectories:

| Folder | Contents |
| --- | --- |
| scripts\ | Python scripts (gcms_pipeline.py, cli.py, src/) |
| software\mzmine\ | MZmine portable installation |
| config\ | MZmine batch file (.mzbatch) |
| users\ | MZmine user file (.mzuser) |
| libraries\ | Spectral library files (.csv) |
| temp\ | Temporary files during processing |

## Installation Steps

### Step 1: Download and Place Files

- Download gcms_pipeline.py, run_pipeline.bat, and setup_env.bat

- Place gcms_pipeline.py in the scripts\ folder

- Place run_pipeline.bat and setup_env.bat in the pipeline root folder

### Step 2: Install MZmine

- Download MZmine 4 portable (ZIP) from mzmine.github.io

- Extract to software\mzmine\

- Verify mzmine_console.exe exists in this folder

### Step 3: Create MZmine User File

MZmine requires authentication for command-line use. Open a command prompt and run:

cd /d X:\Data\ChrisB\Non-Target\gcms_pipeline

software\mzmine\mzmine_console.exe -login-console

Follow the prompts to log in. Then copy the created .mzuser file to the users\ folder.

### Step 4: Create MZmine Batch File

- Open MZmine GUI: software\mzmine\mzmine.exe

- Go to mzwizard menu and select GC-EI workflow

- Configure parameters for your instrument (see recommended settings below)

- Save the batch file to config\gc_ei_workflow.mzbatch

### Step 5: Set Up Python Environment

Open Anaconda Prompt (or Command Prompt) and run:

cd /d X:\Data\ChrisB\Non-Target\gcms_pipeline

setup_env.bat

This creates a virtual environment and installs required Python packages.

### Step 6: Verify Configuration

Edit the paths at the top of scripts\gcms_pipeline.py if your setup differs from the defaults:

PIPELINE_ROOT = Path(r"X:\Data\ChrisB\Non-Target\gcms_pipeline")

MSCONVERT = Path(r"C:\Users\cbrunet\AppData\Local\OpenMS-3.1.0\...\msconvert.exe")

DEFAULT_LIBRARY = PIPELINE_ROOT / "libraries" / "unified_library_20251013.csv"

# How to Use

The pipeline supports three entry points depending on your starting data:

## Entry Point 1: From Raw Agilent Files

Use this when starting with Agilent .D folders directly from the instrument. This runs the complete pipeline: conversion → MZmine → library matching.

run_pipeline.bat --from-raw "X:\Data\Project\raw_data"

## Entry Point 2: From mzML Files

Use this when you already have converted mzML files. This skips conversion and runs: MZmine → library matching.

run_pipeline.bat --from-mzml "X:\Data\Project\converted"

## Entry Point 3: From MZmine Output

Use this when you already have MZmine results and only want to run library matching. The folder must contain a quantification CSV and MSP spectral file.

run_pipeline.bat --from-mzmine "X:\Data\Project\mzmine_output\run_001"

## Common Usage Examples

### Specify a Project Name

By default, the output folder name is derived from the input folder plus a timestamp. To specify a custom name:

run_pipeline.bat --from-raw "X:\Data\raw" --name "NYC_Winter_2024"

### Adjust Processing Threads

If you encounter memory errors, reduce threads. If you have plenty of RAM, increase them:

run_pipeline.bat --from-raw "X:\Data\raw" --threads 1    # Low memory

run_pipeline.bat --from-raw "X:\Data\raw" --threads 4    # High memory

### Use a Custom Library

run_pipeline.bat --from-raw "X:\Data\raw" --library "X:\Libraries\custom.csv"

### Change Blank Sample Identifier

The library matching uses blank subtraction. By default it looks for samples containing "fieldblank". To change this:

run_pipeline.bat --from-raw "X:\Data\raw" --blank-id "hex"

# Command-Line Options

## Entry Point Arguments (Required)

You must specify exactly one of these:

| Argument | Description |
| --- | --- |
| --from-raw, -r | Path to folder containing Agilent .D files. Runs full pipeline. |
| --from-mzml, -m | Path to folder containing mzML files. Skips conversion step. |
| --from-mzmine, -z | Path to folder with MZmine output (CSV + MSP). Runs matching only. |

## Optional Arguments

| Argument | Description | Default |
| --- | --- | --- |
| --name, -n | Project name for output folders | folder + timestamp |
| --threads, -t | Number of MZmine processing threads | 2 |
| --library, -l | Path to spectral library CSV file | unified_library_20251013.csv |
| --blank-id, -b | String to identify blank samples (case-insensitive) | fieldblank |
| --ri-cal | RI calibration file (tab-delimited: Carbon#, RT) | None |
| --api-key, -k | EPA CompTox API key for toxicity data | None |
| --verbose, -v | Enable verbose error output | False |

# Output Files

Results are organized into three output directories:

| Location | Contents |
| --- | --- |
| converted\<run_name>\ | Converted mzML files from msconvert |
| mzmine_output\<run_name>\ | Feature tables (CSV) and deconvoluted spectra (MSP) |
| results\<run_name>\ | Library matching results (CSV and PDF reports) |

# Troubleshooting

## Out of Memory Errors

If you see "Java heap space" errors during MZmine processing:

- Reduce threads: --threads 1

- Process fewer files at once (split into smaller batches)

- Ensure temp directory is on a drive with sufficient space

## MZmine Won't Start

- Verify the .mzuser file exists in the users\ folder

- Re-run: software\mzmine\mzmine_console.exe -login-console

- Check that mzmine_console.exe (not mzmine.exe) is being used

## No .D Files Found

- Ensure the path points to the folder containing .D directories

- Check capitalization (.D vs .d) - the script handles both

## Library Matching Fails

- Verify the library file exists at the specified path

- Check that MZmine output contains both CSV and MSP files

- Ensure cli.py and src/ folder are in the scripts\ directory

## Conversion Fails

- Verify msconvert.exe path in the script configuration

- Try running msconvert directly on a single file to test

# Recommended MZmine Settings

These settings are optimized for Agilent 7250 GC-QTOF data:

| Parameter | Recommended Value |
| --- | --- |
| Noise threshold (MS1) | 500 (5.0E2) |
| Minimum feature height | 1,000 - 10,000 |
| m/z tolerance (scan-to-scan) | 0.005 m/z or 20 ppm |
| m/z tolerance (sample-to-sample) | 0.004 m/z or 8 ppm |
| RT tolerance (intra-sample) | 0.04 min |
| RT tolerance (sample-to-sample) | 0.10 min |
| Min signals in deconvoluted spectrum | 8 |

For questions or issues, contact the pipeline maintainer.